Air Quality

Not for Public Distribution-
for Internal Review Only

### Affected Environment

#### Introduction

The Old River site is located on the boundary between Contra Costa County, which is in the San Francisco Bay Area Air Basin, and San Joaquin County, which is in the San Joaquin Valley Air Basin. These air basins are under the jurisdiction of the Bay Area Air Quality Management District (BAAQMD) and the San Joaquin Valley Air Pollution Control District (SJVAPCD), respectively. The Connection Slough site is wholly in the San Joaquin Valley Air Basin, as is the Roberts Island #1 disposal site.

State and federal laws define criteria emissions to include the following: reactive or volatile organic compounds (ROC or VOC), nitrogen dioxide (NO2), carbon monoxide (CO), sulfur dioxide (SO2), respirable particulate matter (PM10), and fine particulate matter (PM2.5). During the installation of the proposed components, the Proposed Action would temporarily cause criteria emissions from the combustion of fossil fuels (i.e., diesel, gasoline) used to run construction equipment and vehicles, both onsite and offsite. Installation activities also would cause emissions of fugitive dust, primarily as PM10. During operations, emissions would result primarily from vehicle trips generated by the gate operations and the potential use of diesel-powered generators at each of the proposed sites. Because the Proposed Action would require that PG&E provide electric power, the generators would be used as back-up source of power. However, it may take some time before PG&E is able to connect the 2-Gates facilities to the electric grid, and the generators would be used until this occurred. The generators would be State-certified under the Portable Equipment Registration Program or permitted pursuant to SJVAPCD regulations.

The potential for impacts on climate change associated with greenhouse gas (GHG) emissions is discussed in Section 3.16.

#### Meteorology

In summer, northwest winds to the west of the Pacific coastline are drawn into the interior through the Golden Gate and over the lower portions of the San Francisco Peninsula. This channeling of the flow through the Golden Gate produces a jet of air that sweeps eastward but widens downstream producing southwest winds at Berkeley and northwest winds at San Jose; a branch curves eastward through the Carquinez Strait and into the Central Valley. In winter, the Bay Area experiences periods of storminess and moderate-to-strong winds and periods of stagnation with very light winds. Winter stagnation episodes are characterized by outflow from the Central Valley, nighttime drainage flows in coastal valleys, weak onshore flows in the afternoon and otherwise light and variable winds (BAAQMD n.d.). Annual average wind speeds in the central Bay Area (San Francisco County) are 8.7 miles per hour (mph) or 3.9 meters per second (m/s). Annual average wind speeds in the Stockton area (San Joaquin County) are 7.5 mph or 3.4 m/s, which is typical for the vicinity of the Proposed Action. The climate is characterized by moderately wet winters and dry summers. About 90 percent of the annual total rainfall is received between November and April period. Between June and September, normal rainfall is typically less than 0.1 inch (BAAQMD n.d.). Temperatures average about 60 degrees Fahrenheit (°F) annually, with summer highs in the 80s and winter lows in the 40s. Precipitation averages about 18 inches per year, although annual precipitation varies markedly from year to year (CSW 2008).

#### Ambient Air Quality

The BAAQMD and SJVAPCD each operate a regional air monitoring network, together comprising over 50 monitoring stations that collectively measure the ambient concentrations of the six criteria air pollutants described above: ozone (O3), NO2, SO2, CO, PM10, and PM2.5.

Not all monitoring stations are fully instrumented for all the above pollutants. For this assessment, BAAQMD’s Bethel Island station data is used as historic and representative since it is located only 4.3 miles northwest (upwind) of the Old River site and monitors all but one pollutant (PM2.5), while SJVAPCD’s Stockton station is 15 miles east (downwind). Existing and probable future air quality in the vicinity of the Proposed Action can generally be inferred from ambient air quality measurements taken at the Bethel Island site. Table 3.3-1 is a six-year summary of historic monitoring data (2002 to 2007) obtained by the Bethel Island station, except for PM2.5. Data on PM2.5 are from the BAAQMD’s Concord monitoring station (BAAQMD 2008).

During the period from 2002 to 2007, there were no daily violations of state or federal ambient air quality standards for nitrogen dioxide, sulfur dioxide, or carbon monoxide recorded at the Bethel Island station (BAAQMD 2008); however, there were exceedences of ozone, PM10 and PM2.5 standards. Table 3.3-2 shows the incidence of daily violations of ambient ozone, PM10 and PM2.5 standards for the six-year period.

| Table 3.3-1 Ambient Air Quality Summary for Bethel Island 2002 to 2007, Maximums |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pollutant | Period | Units | 2007 | 2006 | 2005 | 2004 | 2003 | 2002 |
| Ozone (O3) | 1-hour max | ppmv | 0.093 | 0.116 | 0.089 | 0.100 | 0.090 | 0.110 |
|  | 8-hour max | ppmv | 0.078 | 0.090 | 0.077 | 0.080 | 0.080 | 0.100 |
|  | 3-year avg | ppmv | 0.073 | 0.073 | 0.072 | 0.075 | 0.079 | 0.079 |
| Nitrogen Dioxide (NO2) | 1-hour max | ppmv | 0.048 | 0.044 | 0.038 | 0.030 | 0.050 | 0.040 |
|  | Annual avg | ppmv | 0.008 | 0.008 | 0.007 | 0.008 | 0.009 | 0.010 |
| Sulfur Dioxide (SO2) | 24-hour max | ppmv | 0.005 | 0.007 | 0.006 | 0.006 | 0.006 | 0.009 |
|  | Annual avg | ppmv | 0.002 | 0.002 | 0.002 | 0.002 | 0.002 | 0.003 |
| Carbon Monoxide (CO) | 1-hour max | ppmv | 1.1 | 1.3 | 1.1 | 1.2 | 1.6 | 1.7 |
|  | 8-hour max | ppmv | 0.8 | 1.0 | 0.9 | 0.9 | 0.9 | 1.3 |
| Particulates (as PM10) | 24-hour max | µg/m3 | 49.0 | 84.0 | 64.0 | 42.0 | 51.0 | 58.0 |
|  | Annual avg | µg/m3 | 18.8 | 19.4 | 18.5 | 19.5 | 19.4 | 23.8 |
| Particulates (as PM2.5) | 24-hour max | µg/m3 | 46.2 | 62.1 | 48.9 | 74.0 | 50.0 | 77.0 |
|  | Annual avg | µg/m3 | 8.4 | 9.3 | 9.0 | 10.7 | 9.7 | 13.3 |
| Source: BAAQMD 2008 Notes: Bethel Island, Concord for PM2.5 ppmv = parts per million by volume µg/m3 = micrograms per cubic meter |  |  |  |  |  |  |  |  |

| Table 3.3-2 Ozone, PM10 and PM2.5 Standard Violation Days for Bethel Island, 2002 to 2007 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pollutant | Standard | Total | 2007 | 2006 | 2005 | 2004 | 2003 | 2002 |
| Ozone (O3) | Federal | 4 | 0 | 1 | 0 | 0 | 0 | 3 |
|  | California | 21 | 4 | 14 | 2 | 1 | 0 | 0 |
| Particulates (as PM10) | Federal | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | California | 6 | 0 | 1 | 1 | 0 | 1 | 3 |
| Particulates (as PM2.5) | Federal | 17 | 7 | 5 | 0 | 1 | 0 | 4 |
|  | California | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Source: BAAQMD 2008 |  |  |  |  |  |  |  |  |

#### Sensitive Receptors

Certain population groups are considered more sensitive to air pollution and odors than others, particularly children, elderly, and acutely ill and chronically ill persons, especially those with cardio-respiratory diseases such as asthma and bronchitis. Sensitive receptors (land uses) indicate locations where such individuals are typically found; e.g., schools, daycare centers, hospitals, convalescent homes, residences of sensitive persons, and parks with active recreational uses, such as youth sports.

Persons engaged in strenuous work or physical exercise also have increased sensitivity to poor air quality. Residential areas are considered more sensitive to air quality conditions than commercial and industrial areas because people generally spend longer periods of time at their residences, resulting in greater exposure to ambient air quality conditions. Recreational uses such as parks are also considered sensitive due to the greater exposure to ambient air quality conditions and because the presence of pollution detracts from the recreational experience.

The Old River, Connection Slough, and Roberts Island #1 disposal sites are located in sparsely populated rural (agricultural) areas. The nearest house is approximately 600 feet (180 meters) south of the Old River site; however, it is unoccupied. The next nearest receptor to the proposed gate sites is a marina with live-aboard boat owners approximately 0.8 to 1 miles (1,300 to 1,600 meters) south of the Old River site.

### Regulatory Setting

#### State and National Ambient Air Quality Standards

The Clean Air Act of 1970 (CAA), (as amended 1977 and 1990, 42 U.S.C. 7401 et seq.) established national ambient air quality standards (NAAQS) and delegates the enforcement of these standards to the states. In California, the California Air Resources Board (CARB) is responsible for enforcing air pollution regulations. The CARB has in turn delegated the responsibility of regulating stationary emission sources to local air agencies (i.e., BAAQMD and SJVAPCD). In areas that exceed the NAAQS, the CAA requires preparation of a State Implementation Plan (SIP), detailing how the states will attain the standards within mandated time frames. As shown in Table 3.3-3, California ambient air quality standards (CAAQS) tend to be at least as protective as national standards and are often more stringent.

Air districts in California are required to monitor air pollutant levels to assure that NAAQS and CAAQS are met and, in the event that they are not, to develop strategies to meet these standards. Depending on whether the standards are met or exceeded, the local air basin is classified as being in “attainment” or “non-attainment.”

The air pollutants of most concern in California are ozone and particulate matter. The San Francisco Bay Area Air Basin (including Contra Cost County) and the San Joaquin Valley Air Basin (including San Joaquin County) are in NAAQS attainment except for the following federal standards shown in Table 3.3-3:

- 8-hour ozone – Non-attainment for both the San Francisco and San Joaquin Valley Air Basins (CARB 2006b).

- 24-hour PM10 – Unclassified for the San Francisco Bay Area Air Basin (CARB 2009).

- 24-hour PM2.5 – Non-attainment for San Joaquin Valley Air Basin (San Francisco Bay Area Air Basin - Unclassified) (CARB 2006c).

- The San Francisco Bay Area Air Basin is a “Marginal” area for federal 8-hour ozone and originally had to attain the now revoked federal 1-hour ozone standard by 1999.

- The San Joaquin Valley Air Basin is presently a “Serious” and a pending “Severe 17” area for federal 8-hour ozone and nevertheless plans to attain the now revoked federal 1-hour ozone standard by 2010 (see below).

On April 30, 2007, the Governing Board of the SJVAPCD voted to request the U.S. Environmental Protection Agency (EPA) to reclassify the San Joaquin Valley Air Basin as “Extreme” (now referred to as “Severe 17”) non-attainment for the federal 8-hour ozone standards. The CARB, on June 14, 2007, approved this request. This request must be forwarded to EPA by the CARB and would become effective upon EPA final rulemaking after a notice and comment process; it is not yet in effect (SJVAPCD 2007).

Effective June 15, 2005, the EPA revoked in the federal 1-hour ozone standard, including associated designations and classifications. However, EPA had previously classified the SJVAB as extreme nonattainment for this standard. Many applicable requirements for extreme 1-hour ozone nonattainment areas continue to apply to the SJVAB (SJVAPCD 2005).

| Table 3.3-3 State and Federal Ambient Air Quality Standards |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Pollutant | Averaging Time | California Standards |  | Federal Standards |  |
|  |  | ppmv | µg/m3 | ppmv | µg/m3 |
| Ozone (O3) | 1-hour | 0.09 | 177 | -- | -- |
|  | 8-hour | 0.07 | 137 | 0.075 | 147 |
| Nitrogen Dioxide (NO2) | 1-hour | 0.18 | 338 | -- | -- |
|  | Annual | 0.03 | 56 | 0.053 | 100 |
| Sulfur Dioxide (SO2) | 1-hour | 0.25 | 655 | -- | -- |
|  | 3-hour (secondary) | -- | -- | 0.50 | 1,309 |
|  | 24-hour | 0.04 | 105 | 0.14 | 367 |
|  | Annual | -- | -- | 0.03 | 79 |
| Carbon Monoxide (CO) | 1-hour | 20 | 22,898 | 35 | 40,071 |
|  | 8-hour | 9 | 10,304 | 9 | 10,304 |
|  | Lake Tahoe (8-hour) | 6 | 6,869 | -- | -- |
| Particulates (as PM10) | 24-hour | -- | 50 | -- | 150 |
|  | Annual | -- | 20 | -- | -- |
| Particulates (as PM2.5) | 24-hour | -- | -- | -- | 35 |
|  | Annual | -- | 12 | -- | 15 |
| Lead (Pb) | 30-day | -- | 1.5 | -- | -- |
|  | 90-day | -- | -- | -- | 1.5 |
| Sulfates (as SO4) | 24-hour | -- | 25 | none | none |
| Hydrogen Sulfide (H2S) | 1-hour | 0.03 | 42 | none | none |
| Vinyl Chloride (C2H3CI) | 24-hour | 0.01 | 26 | none | none |
| Visibility Reducing Particles | 8-hour | Extinction coefficient of 0.23 per kilometer; visibility of 10 miles or more (0.07 to 30 miles or more for Lake Tahoe) due to particles when relative humidity is less than 70%. |  | none | None |
| Source: CARB 2008 Notes: Standard Temperature = 25ºC Standard Molar Volume = 24.465 liter/gram-mole For gases, μg/m3 calculated from ppmv based on molecular weight and standard conditions ppmv = parts per million by volume μg/m3 = micrograms per cubic meter |  |  |  |  |  |

The San Francisco Bay Area and San Joaquin Valley Air Basins are in CAAQS attainment except for the following state standards shown in Table 3.3-3 (BAAQMD 2008, SJVAPCD 2008):

| - 8-hour ozone – Non-attainment | - Annual PM2.5 – Non-attainment |
| --- | --- |
| - 1-hour ozone – Non-attainment | - 1-hour hydrogen sulfide - Unclassified |
| - Annual PM10 – Non-attainment | - 8-hour visibility reducing particles – Unclassified |
| - 24-hour PM10 – Non-attainment |  |

Similar to the federal CAA, the California CAA also classifies areas according to pollution levels. Under the California CAA, the San Francisco Bay Area Air Basin is a “Serious” state ozone non-attainment area and a state PM10 and PM2.5 non-attainment area. The San Joaquin Valley Air Basin is presently a “Severe” state ozone non-attainment area, in addition to being a state PM10 and PM2.5 non-attainment area.

#### Regional Plans

For the San Francisco Bay Area Air Basin, the Association of Bay Area Governments, the Metropolitan Transportation Commission, and BAAQMD jointly prepare the Bay Area Clean Air Plan and Ozone Attainment Plan (BAAQMD 2000, 2001).

For the San Joaquin Valley Air Basin, the Extreme Ozone Attainment Demonstration Plan is prepared by the SJVAPCD, in conjunction with the CARB, the EPA, and the eight regional Transportation Planning Agencies (SJVAPCD 2005).

These plans contain control strategies that demonstrate attainment with the national ambient air quality standards by the deadlines established in the CAA.

#### Air Toxics Control Measures

On July 26, 2007, the CARB adopted a regulation to reduce diesel particulate matter and nitrogen oxide emissions from in use (existing) off-road heavy-duty diesel vehicles in California. The regulation will require fleet owners to accelerate turnover to cleaner engines and install exhaust retrofits.

#### Senate Bill 656

Senate Bill (SB) 656 is a planning requirement that calls for a plan and strategy for reducing PM2.5 and PM10. This bill requires the CARB to identify, develop, and adopt a list of control measures to reduce the emissions of PM2.5 and PM10 from new and existing stationary, mobile, and area sources. The BAAQMD and SJVAPCD have developed particulate matter control measures and submitted plans to the CARB that include lists of measures to reduce particulate matter. Under the plans, the Districts are required to continue to assess PM2.5 and PM10 emissions and their impacts. For construction emissions of fugitive PM10, the Districts have adopted a number of feasible control measures that can be reasonably implemented to significantly reduce fugitive PM10 emissions from construction. In general, the Districts’ approach to the analyses of construction impacts is to emphasize implementation of effective and comprehensive control measures rather than detailed quantification of emissions.

#### Toxic Air Contaminants

A project with the potential to expose sensitive receptors (including residential areas) or the general public to substantial levels of toxic air contaminants, as designated by the CARB under 17 CCR Section 93001, listed in the BAAQMD 2003 Annual Report Appendix A: Toxic Air Contaminants (BAAQMD 2003), and similarly, in the SJVAPCD 2006 Annual Report on the District’s Toxics Program (SJVAPCD 2006), would be deemed to have a significant impact. This includes projects that would locate receptors near existing sources of toxic air contaminants, as well as projects that would place sources of toxic air contaminants near existing receptors.

Proposed projects that have the potential to expose the public to toxic air contaminants in excess of the following thresholds would be considered to have a significant air quality impact. These thresholds, which are based on BAAQMD Regulation 2, Rule 5, New Source Review of Toxic Air Contaminants and SJVAPCD (2002) Assessment Guidance, are as follows:

- Probability of contracting cancer for the Maximally Exposed Individual (MEI) exceeds ten in one million. The MEI is a hypothetical person exposed for 70 years continuously (24 hours per day, 365 days per year).

- Ground-level concentrations of non-carcinogenic toxic air contaminants would result in a Hazard Index greater than one for the MEI.

Diesel particulate matter is considered a toxic air contaminant in California (BAAQMD 2003, SJVAPCD 2006). The impact assessment includes a screening-level Health Risk Assessment for diesel particulate matter impacts on sensitive receptors from construction equipment.

#### General Conformity

Section 176(c) of the CAA contains the General Conformity Rule (40 CFR 51.850-860 and 40 CFR 93.150-160). The General Conformity Rule requires that a federal agency responsible for a proposed action (e.g., the 2-Gates Project) in a NAAQS non-attainment or maintenance area endeavor to ensure that the proposed action conforms to the applicable SIP. This means that federally supported or funded activities shall not: 1) cause or contribute to any new air quality standard violation, 2) increase the frequency or severity of any existing standard violation, or 3) delay the timely attainment of any standard, interim emission reduction, or other milestone. Emissions of attainment pollutants are exempt from the General Conformity Rule. A federal action would comply with an applicable SIP if it does not exceed identified annual emission de minimis thresholds, the magnitudes of which are based on the severity of the non-attainment rating of the region in which the Proposed Action is located. Actions that exceed these thresholds are required to conduct in depth conformity determinations.

Contra Costa and San Joaquin counties are in federal and state non-attainment for ozone, PM10, and PM2.5. Thus, the emissions of non-attainment pollutants NOX, ROC, PM10, and PM2.5 would be subject to the General Conformity Rule. As discussed below under Environmental Consequences, emissions from the Proposed Action would be below BAAQMD and SJVAPCD annual thresholds for non-attainment pollutants; thus, the de minimis requirement is satisfied.

### Environmental Consequences

#### No Action Alternative

No air quality impacts would result from the No Action alternative because no construction would occur.

#### Proposed Action

The only source of direct emissions during operation of the Proposed Action would be associated with vehicle trips required during infrequent periodic inspections and maintenance activities, personal vehicle trips by the gate operators when the gates are being operated, and the potential temporary use of portable generators at the Old River and Connection Slough sites until power could be obtained from PG&E. Emissions from these sources would be minor and intermittent and would not result in permanent air quality impacts, nor would they require permits from the BAAQMD or the SJVAPCD. Any impacts from operations would be negligible. The impact assessment focuses on the emissions that would occur as a result of proposed construction activities because these are the main source of emissions.

##### Methodology

Construction emissions fall into three general categories: 1) onsite use of diesel-powered construction equipment, 2) onsite controlled (mitigated) fugitive dust generation from demolition and earthmoving activities, and 3) offsite vehicle traffic comprising project-related trucking and project worker commuting. Construction-related emissions are generally short-term in duration, but may still cause localized adverse air quality impacts. Specific to this type of project, dredging and pile driving equipment would be permitted pursuant to SJVAPCD regulations.

The analysis of the Proposed Action’s air quality impacts is based on equipment specifications and planning estimates for the construction (installation) phase of the Proposed Action as listed in Tables 3.3-4 and 3.3-5, respectively. A detailed air impact analysis associated with the complete removal of all components at the end of the demonstration period is not included because emissions would be less than those required for installation (the removal period would last only 4 weeks, as opposed to approximately 21 weeks for construction, less equipment would be required [e.g., no dredging would be needed], and some of the rock would be left in the channel bottom, thus requiring less movement of materials).

| Table 3.3-4 Construction Estimated Equipment List |  |  |  |
| --- | --- | --- | --- |
| Equipment Type | Manufacturer/Model | Quantity | Horsepower |
| Off-Road Construction (onsite) |  |  |  |
| Loader | CAT 966G | 2 | 233 |
| Forklift | CAT TH83 | 2 | 109 |
| Excavator | CAT 330 | 2 | 268 |
| On-Road Vehicles (offsite) |  |  |  |
| Grove Boom Truck RT 522 | RT 522 | 1 | 150 |
| Flat Bed Truck | Chevy | 1 | 250 |
| Pick Up Truck | Chevy/Silverado | 4 | 200 |
| Fuel/Service Truck | Kenworth | 1 | 225 |
| Water Truck (3600 gallons) | Kenworth | 1 | 400 |
| Marine Vessels and Equipment (onsite) |  |  |  |
| DB 24 (with Amclyde 28 crane) | CAT 3412 | 1 | 525 |
| Dump Scow 5 (hopper barge) | CAT 3208 | 2 | 210 |
| CB 8 (with Bucyrus-Erie 88B crane) | Cummins V1710 | 1 | 365 |
| CB Doolittle (with Bucyrus-Erie 65D crane) | Cummins 855 | 1 | 280 |
| Workboat | John Deer 400 | 3 | 600 |
| Tugboat "Sarah Reed" | Cummins KTA38 | 2 | 1700 |
| Generator 25KW | Rental | 4 | 35 |
| Vibratory / Impact Hammer | APE 200/CAT C16 | 1 | 630 |
| Flat Deck Material Barge | n/a | 6 |  |
| Dredged Material Disposal (offsite) |  |  |  |
| Tugboat for Dredged Material Barge | Charter | 2 | 800 |
| Offloading Crane | Rental | 1 | 750 |
| D6 Dozer | Caterpillar D6 | 1 | 200 |
| Barge for Dredged Material | n/a | 3 |  |
| Source: Dutra Group 2008 (updated 2009) (see Appendix G) |  |  |  |

| Table 3.3-5 Construction Planning Estimate |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Project Activity | Schedule |  |  |  |  |
|  | hours/day | days/week | weeks | days | hours |
| Land-Side Sheet Pile Driving | 8 | 5 | 3.2 | 16 | 128 |
| Submerged Sheet Pile Driving | 12 | 5 | 7.6 | 38 | 456 |
| Dredging & Spoils Disposal | 12 | 5 | 4.4 | 22 | 264 |
| Gravel Bedding Fill Placement | 12 | 5 | 4.6 | 23 | 276 |
| Rock Locking Fill Placement | 12 | 5 | 6.6 | 33 | 396 |
| Vessels Tending (concurrent) | 12 | 5 | 19.4 | 97 | 1164 |
| Totals (with activity overlaps) | 12 | 5 | 19 | 97 |  |
| Source: Moffat & Nichol September 24, 2009 (see Appendix G) |  |  |  |  |  |

Combustion Emissions. Table 3.3-6 shows estimated maximum fuel consumption for the Proposed Action based on equipment specifications and planning estimates for the site preparation and construction activities provided by the contractor, assuming a brake specific fuel consumption (BSFC) of 0.051 gallons (gal) per brake horsepower-hour (BHP-hr) (AP-42, Table 3.3-1) (EPA 2006). If actual fuel consumption is lower, there would be correspondingly lower emissions. California ultra-low sulfur diesel fuel with a maximum sulfur content of 15 parts per million (ppm) by weight would be used in all diesel-powered equipment to minimize sulfur dioxide and particulate emissions.

| Table 3.3-6 Estimated Maximum Fuel Consumption |  |  |  |
| --- | --- | --- | --- |
| Project Activity | Hourly | Daily | Project |
|  | gal/hour | gal/day | gallons |
| Onsite Preparation and Construction | 130 | 1,400 | 76,000 |
| Offsite Preparation | 80 | 990 | 19,900 |
| Maximum Impact Totals | 130 | 1,400 | 95,900 |
| Source: M&N September 24, 2009 (see Appendix G) |  |  |  |
| Notes: |  |  |  |
| Maximum onsite hourly and daily rates for concurrent activities |  |  |  |
| BSFC = (7,000 British thermal units [BTU]/BHP-hr) / (137,030 BTU/gal) = 0.051 gal/BHP-hr |  |  |  |
| AP-42 Table 3.3-1 (EPA 2006) |  |  |  |

Combustion emissions were estimated using the emission factors given in Table 3.3-7 for diesel nonroad equipment. For calculating emissions, EPA Tiered emission factors (40 CFR 89.112 & 13 CCR 2423) in grams per BHP-hr were converted to pounds per thousand gallons (lb/mgal) burned, assuming a diesel default heat rate of 7,000 British thermal units (BTU) per BHP-hr and a higher heating value of 137,030 BTU per gallon (AP-42, Table 3.3-1) (EPA 2006). Average engine age (Tier) was estimated based on Annex 3, Table A-101 and Table A-84, Inventory of U.S. Greenhouse Gas Emissions and Sinks: 1990-2007 (EPA 2009). The age analysis yielded an estimated distribution of 31 percent Tier 3, 28 percent Tier 2, 19 percent Tier 1, and 22 percent Uncontrolled for 2010. The use of newer, less polluting Tier 1, 2, and 3 engines in the majority of construction equipment used onsite is a mitigating factor for combustion emissions of NOX, ROC, CO, PM10, and PM2.5.

Fugitive Dust Emissions. PM10 in the form of fugitive dust is the pollutant of greatest concern with respect to construction activities. Fugitive PM10 emissions can result from a variety of construction activities, including excavation, grading, demolition, vehicle travel on paved and unpaved surfaces, and vehicle and equipment exhaust. Construction-related emissions, including site preparation, can cause substantial increases in localized concentrations of PM10. Particulate emissions from construction activities can lead to adverse health effects as well as nuisance concerns such as reduced visibility and soiling of exposed surfaces.

| Table 3.3-7 Tiered Nonroad Diesel Emission Factors, Pounds per 1000 Gallons |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Emittent | Pre-control lb/mgal | Tier 1 (96) lb/mgal | Tier 2 (01) lb/mgal | Tier 3 (06) lb/mgal | Composite lb/mgal |
| Oxides of Nitrogen (as NO2) | 604.2 | 297.8 | 181.3 | 112.2 | 276.2 |
| Hydrocarbons (ROC as CH4) | 47.5 | 43.2 | 25.9 | 17.3 | 30.2 |
| Carbon Monoxide (CO) | 129.5 | 366.8 | 112.2 | 112.2 | 164.0 |
| Particulates (as PM10) | 43.2 | 17.3 | 6.5 | 6.5 | 17.3 |
| Sulfur Dioxide (SO2) | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| Carbon Dioxide (GHG - CO2) | 22,485 | 22,485 | 22,485 | 22,485 | 22,485 |
| Nitrous Oxide (GHG - N2O) | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |
| Methane (GHG - CH4) | 1.3 | 1.3 | 1.3 | 1.3 | 1.3 |
| Source: Inventory of U.S. Greenhouse Gas Emissions and Sinks: 1990-2007, EPA 2009; 40 CFR 89.112; 13 CCR 2423 Notes: Nonroad Tier 1, 2, 3 per 40 CFR 89.112 & 13 CCR 2423 Precontrol NOX, ROC, CO, PM10 per AP-42 Table 3.3-1 2010 engine age profile estimation based on Annex 3, Table A-101, Table A-84, US GHG Inventory 22% Precontrol (uncontrolled) 19% Tier 1 28% Tier 2 31% Tier 3 Default heat rate = 7,000 BTU/BHP-hr (AP-42 Table 3.3-1) Diesel = 19,300 BTU/lb, 7.1 lb/gal (AP-42 Table 3.3-1) |  |  |  |  |  |

Construction areas on Bacon Island, the Holland Tract, and Connection Slough would comprise 4.13, 4.13, and 2.75 acres, respectively, for a total of 11.02 acres. These areas were used to estimate fugitive dust emissions using the BAAQMD and SJVAPCD protocol described below. Offsite disposal of excess dredged material on Roberts Island could also involve an area of up to 4.13 acres.

Construction emissions of fugitive PM10 can vary greatly depending on the level of activity, the specific operations taking place, the equipment being operated, local soils, weather conditions, and other factors. Despite this variability in emissions, experience has shown that there are a number of feasible control measures that can be reasonably implemented to significantly reduce fugitive PM10 emissions from construction. The Districts’ approach to environmental analyses of construction impacts is to emphasize implementation of effective and comprehensive control measures rather than detailed quantification of emissions.

For land disturbance, fugitive dust (as PM10) was estimated as 51 pounds per acre per day unmitigated (uncontrolled) as specified in the BAAQMD guidelines (BAAQMD 1999), consistent with SJVAPCD Assessment Guidelines (SJVAPCD 2002), Section 3.3; AP-42 Chapter 13.2.3 “Heavy Construction Operations”; and AP-42 Chapter 13.2.2 “Unpaved Roads,” Figure 13.2.2-2 (EPA 2006). For the BAAQMD and SJVAPCD control measures listed in Section 3.3.3.3 below, an equivalent soil to moisture ratio of 5:1 was assumed for all feasible measures, which reduces fugitive dust emissions by 95 percent from uncontrolled levels.

Offsite Vehicle Emissions. A relatively small source of emissions compared to onsite equipment, offsite vehicle emissions consist of worker commute trips in light-duty vehicles (passenger cars and light trucks) to and from the proposed sites, and heavy-duty truck emissions generally associated with hauling away debris and transporting materials and equipment to the site. Commuter trip estimates were developed using the generalized emissions estimation methodology given in the BAAQMD (1999) guidelines, Section 3.4, Tables 10 and 11. Similarly, heavy-duty truck trip estimates were developed and translated into emissions utilizing CARB’s EMFAC 2007 computer program (i.e., determination of emission factors).

Dispersion Modeling. For onsite emissions, EPA’s SCREEN Version 96043 (EPA 1992) was used to model the Gaussian dispersion of emissions to obtain ambient impacts. For combustion emissions from construction equipment, a single equivalent point source (stack) was modeled to yield maximum potential downwind impact from the construction site, which is highly conservative and thus tends to overestimate impacts. Fugitive dust emissions were modeled as an equilateral area source with zero release height, which is also conservative and thus tends to overestimate impacts. For screening dispersion modeling, the annual average wind speed of 3.6 m/s (NOAA 2008) was assumed for neutral Stability Class D.

Appendix G, Air Quality Calculations, includes detailed calculation and modeling templates.

### Environmental Consequences

The Proposed Action would generate PM10 and PM2.5, primarily through fugitive dust (PM10) emissions during construction activities, and from PM10 and PM2.5 emissions from diesel-powered construction equipment. The BAAQMD and SJVAPCD significance criteria for ozone precursors (NOX and ROC) and PM10 emitted from proposed activities are shown in Tables 3.3-8 and 3.3-9, respectively. For CO emissions, significance is defined as causing a violation of the state standard for CO of 9 ppm averaged over 8 hours or 20 ppm for 1 hour (BAAQMD 1999, SJVAPCD 2002).

| Table 3.3-8 BAAQMD Thresholds |  |  |
| --- | --- | --- |
| Significance Criteria | Total Project |  |
|  | tons/year | lbs/day |
| Oxides of Nitrogen (as NO2) | 15 | 80 |
| Hydrocarbons (ROC as CH4) | 15 | 80 |
| Particulates (as PM10) | 15 | 80 |
| Carbon Monoxide (CO) | Violation of CAAQS for CO |  |
| Source: BAAQMD CEQA Guidelines, Table 3 (BAAQMD 1999) |  |  |

| Table 3.3-9 SJVAPCD Thresholds |  |  |
| --- | --- | --- |
| Significance Criteria | Total Project |  |
|  | tons/year | lbs/day |
| Oxides of Nitrogen (as NO2) | 10 | n/a |
| Hydrocarbons (ROC as CH4) | 10 | n/a |
| Carbon Monoxide (CO) | Violation of CAAQS for CO |  |
| Source: Guide for Assessing and Mitigating Air Quality Impacts, Table 3-1 (SJVAPCD 2002) |  |  |

A preliminary screening impact analysis was performed, estimating the controlled onsite, offsite, and total emissions from construction activities. The results are summarized in Tables 3.3-10, 3.3-11, and 3.3-12, respectively.

| Table 3.3-10 Estimated Onsite Construction Criteria Emissions, Controlled |  |  |  |
| --- | --- | --- | --- |
| Project Emissions | tons | lb/day | lb/hour |
| Oxides of Nitrogen (as NO2) | 10.50 | 385.30 | 35.33 |
| Hydrocarbons (ROC as CH4) | 1.15 | 42.13 | 3.86 |
| Carbon Monoxide (CO) | 6.23 | 228.78 | 20.98 |
| Particulates (as PM10) | 0.66 | 24.13 | 2.21 |
| Sulfur Dioxide (SO2) | 0.01 | 0.28 | 0.03 |
| Carbon Dioxide (GHG - CO2) | 854 | 31,366 | 2,876 |
| Nitrous Oxide (GHG - N2O) | 0.02 | 0.84 | 0.08 |
| Methane (GHG - CH4) | 0.05 | 1.81 | 0.17 |
| Diesel Particulate Matter (DPM) | 0.66 | 24.13 | 2.21 |
| Fugitive Dust (as PM10) | 1.36 | 28.10 | 2.34 |

| Table 3.3-11 Estimated Offsite Construction Criteria Emissions, Controlled |  |  |  |
| --- | --- | --- | --- |
| Project Emissions | tons | lb/day | lb/hour |
| Oxides of Nitrogen (as NO2) | 3.19 | 283.49 | 25.25 |
| Hydrocarbons (ROC as CH4) | 0.36 | 31.31 | 2.85 |
| Carbon Monoxide (CO) | 2.67 | 184.29 | 19.25 |
| Particulates (as PM10) | 0.29 | 19.66 | 2.06 |
| Sulfur Dioxide (SO2) | 0.01 | 0.37 | 0.06 |
| Carbon Dioxide (GHG - CO2) | 343 | 24,795 | 2,483 |
| Nitrous Oxide (GHG - N2O) | 0.01 | 0.63 | 0.06 |
| Methane (GHG - CH4) | 0.02 | 1.39 | 0.13 |
| Diesel Particulate Matter (DPM) | 0.18 | 17.33 | 1.47 |
| Fugitive Dust (as PM10) | 3.28 | 75.91 | 17.22 |

| Table 3.3-12 Estimated Total Construction Criteria Emissions, Controlled |  |  |  |
| --- | --- | --- | --- |
| Project Emissions | tons | lb/day | lb/hour |
| Oxides of Nitrogen (as NO2) | 13.69 | 668.79 | 60.58 |
| Hydrocarbons (ROC as CH4) | 1.51 | 73.44 | 6.72 |
| Carbon Monoxide (CO) | 8.90 | 413.07 | 40.23 |
| Particulates (as PM10) | 0.95 | 43.79 | 4.27 |
| Sulfur Dioxide (SO2) | 0.02 | 0.65 | 0.09 |
| Carbon Dioxide (GHG - CO2) | 1,197 | 56,161 | 5,359 |
| Nitrous Oxide (GHG - N2O) | 0.03 | 1.46 | 0.13 |
| Methane (GHG - CH4) | 0.07 | 3.20 | 0.30 |
| Diesel Particulate Matter (DPM) | 0.84 | 41.47 | 3.69 |
| Fugitive Dust (as PM10) | 4.64 | 104.01 | 19.56 |

Although no BAAQMD or SJVAPCD annual thresholds would be exceeded, daily emissions of NOX and combined daily emissions of PM10 (i.e., combustion plus fugitive dust) are over the BAAQMD threshold. Since Contra Costa and San Joaquin counties are in non-attainment for PM10 and PM2.5, screening dispersion modeling was performed to determine whether state or federal ambient air quality standards would be exceeded solely due to proposed activities against historic maximum background levels, including carbon monoxide. The screening air quality impacts are shown in Table 3.3-13. A screening risk evaluation for diesel particulate matter for the construction period is shown in Table 3.3-14.

The results of the screening analysis for criteria pollutants show that no exceedence of ambient air quality standards in the vicinity of the Proposed Action would result solely from proposed activities. Notwithstanding impacts from the Proposed Action, maximum background levels of particulate matter (PM10, PM2.5) already exceed state or federal standards as applicable in the vicinity. Therefore, the Proposed Action would contribute to these existing exceedences. The BAAQMD and SJVAPCD developed the following emission control measures for construction emissions that, when implemented, would prevent significant contributions of emissions thus mitigating impacts.

| Table 3.3-13 Estimated Onsite Construction Criteria Maximum Impacts, Controlled |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Criteria Pollutant | Averaging Period | Modeled | Back-ground | Total | California Standard |  | Federal Standard |  |
|  |  | ug/m3 | ug/m3 | ug/m3 | ug/m3 | status | ug/m3 | status |
| Nitrogen Dioxide (NO2) | 1-hour max | 16.8 | 94 | 111 | 338 | Under | --- | Under |
|  | Annual avg | 0.4 | 19 | 19 | 56 | Under | 100 | Under |
| Sulfur Dioxide (SO2) | 1-hour max | 0.0 | 59 | 59 | 655 | Under | --- | Under |
|  | 3-hour | 0.0 | 53 | 53 | --- | Under | 1309 | Under |
|  | 24-hour | 0.0 | 24 | 24 | 105 | Under | 367 | Under |
|  | Annual avg | 0.0 | 7 | 7 | --- | Under | 79 | Under |
| Carbon Monoxide (CO) | 1-hour max | 19.5 | 1,946 | 1,966 | 22,898 | Under | 40,071 | Under |
|  | 8-hour | 13.7 | 1,488 | 1,502 | 10,304 | Under | 10,304 | Under |
| Particulates (as PM10) | 24-hour | 0.41 | 84.0 | 84.4 | 50 | Exceed | 150 | Under |
|  | Annual avg | 0.04 | 23.8 | 23.8 | 20 | Exceed | --- | Under |
| Particulates (as PM2.5) | 24-hour | 0.41 | 77.0 | 77.4 | --- | Under | 35 | Exceed |
|  | Annual avg | 0.04 | 13.3 | 13.3 | 12 | Exceed | 15 | Under |
| Fugitive Dust (as PM10) | 24-hour | 18.37 | 84.0 | 102.4 | 50 | Exceed | 150 | Under |
|  | Annual avg | 1.95 | 23.8 | 25.8 | 20 | Exceed | --- | Under |
| Source: BAAQMD 2008, CARB 2008 |  |  |  |  |  |  |  |  |
| Notes: |  |  |  |  |  |  |  |  |
| Background reference is Bethel Island 2002-07 (Concord for PM2.5) |  |  |  |  |  |  |  |  |
| Combustion emissions maximum impact at 1191 m (3907 ft), point or volume source . |  |  |  |  |  |  |  |  |
| Fugitive dust maximum impact at 158 m (518 ft), area source. |  |  |  |  |  |  |  |  |
| ug/m3 = micrograms per cubic meter |  |  |  |  |  |  |  |  |

| Table 3.3-14 Diesel Particulate Matter Screening Health Risk Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Pollutant | Annual | URV | Activity | Annual MEI | Cancer |
|  | ug/m3 | (ug/m3)-1 | days | Correction | Risk |
| Diesel Particulate Matter (DPM) | 0.04 | 3.00E-04 | 97 | 0.0038 | 5.0E-08 |
| Source: California EPA, Office of Environmental Health Hazard Assessment, 2005 |  |  |  |  |  |
| Notes: |  |  |  |  |  |
| Sensitive receptor impact at 1288 m (4226 ft), point or volume source. |  |  |  |  |  |
| ug/m3 = micrograms per cubic meter |  |  |  |  |  |
| URV = Unit Reference (Risk) Value |  |  |  |  |  |

Diesel Emissions Control Measures

The following requirements would be incorporated into contract specifications:

- To minimize potential diesel odor impacts on nearby receptors (pursuant to BAAQMD Regulation 1, Rule 301, and SJVAPCD Regulation IV, Rule 4102, Nuisance), construction equipment will be properly tuned. A schedule of tune-ups will be developed and performed for all equipment. A log of required tune-ups will be maintained and a copy of the log will be submitted to the Project Environmental Compliance Officer for review every 2,000 service hours.

- Fixed temporary sources of air emissions (such as portable pumps, compressors, generators, etc.) will be electrically powered unless the contractor submits documentation and receives approval from the Project Environmental Compliance Officer that the use of such equipment is not practical, feasible, or available (generally contingent upon power line proximity, capacity, and accessibility). California ultra-low sulfur diesel fuel with maximum sulfur content of 15 ppm by weight, or an approved alternative fuel, will be used for onsite fixed equipment not using line power.

- To minimize diesel emission impacts, construction contracts will require off-road compression ignition equipment operators to reduce unnecessary idling with a two-minute time limit.

- On-road and off-road material hauling vehicles will shut off engines while queuing for loading and unloading for time periods longer that two minutes.

- Off-road diesel equipment will be fitted with verified diesel emission control systems (e.g., diesel oxidation catalysts) to the extent reasonably and economically feasible.

- Utilize alternative fuel equipment (i.e., compressed or liquefied natural gas, biodiesel, electric) to the extent reasonably and economically feasible.

Construction emissions of fugitive PM10 can vary greatly depending on the level of activity, the specific operations taking place, the equipment being operated, local soils, weather conditions, and other factors. Despite this variability in emissions, experience has shown that there are a number of feasible control measures that can be reasonably implemented to significantly reduce fugitive PM10 emissions from construction. The Districts’ approach to the analysis of construction impacts is to emphasize implementation of effective and comprehensive control measures rather than detailed quantification of emissions.

##### Dust Control Measures

To control emissions of particulate matter, the Proposed Action would implement the following fugitive dust and particulate matter emissions control measures suggested by the BAAQMD CEQA and SJVAPCD Assessment Guidelines as applicable (BAAQMD 1999, SJVAPCD 2002). The following controls would be implemented at the construction and staging sites as applicable.

- Water all active construction areas at least twice daily as necessary and indicated by soil and air conditions.

- Cover all trucks hauling soil, sand, and other loose materials or require all trucks to maintain at least two feet of freeboard.

- Pave, apply water three times daily, or apply (non-toxic) soil stabilizers on all unpaved access roads, parking areas and staging areas at construction sites.

- Sweep daily (with water sweepers) all paved access roads, parking areas and staging areas at construction sites.

- Sweep streets daily (with water sweepers) if visible soil material is carried onto adjacent public streets.

- All disturbed areas, including storage piles, which are not being actively utilized for construction purposes, will be effectively stabilized of dust emissions using water, chemical stabilizer/suppressant, covered with a tarp or other suitable cover or vegetative ground cover.

- All on-site unpaved roads and off-site unpaved access roads will be effectively stabilized of dust emissions using water or chemical stabilizer/suppressant.

- All land clearing, grubbing, scraping, excavation, land leveling, grading, cut & fill, and demolition activities will be effectively controlled of fugitive dust emissions utilizing application of water or by presoaking.

- When materials are transported off-site, all material will be covered, or effectively wetted to limit visible dust emissions, and at least 6 inches of freeboard space from the top of the container will be maintained.

- All operations will limit or expeditiously remove the accumulation of mud or dirt from adjacent public streets at the end of each workday. The use of dry rotary brushes is expressly prohibited except where preceded or accompanied by sufficient wetting to limit the visible dust emissions. Use of blower devices is expressly forbidden.

- Following the addition of materials to, or the removal of materials from, the surface of outdoor storage piles, said piles will be effectively stabilized of fugitive dust emissions utilizing sufficient water or chemical stabilizer/suppressant.

The estimated effectiveness of these control measures is quantified in Table 3.3-15.

| Table 3.3-15 Estimated Fugitive Dust Emissions from Construction |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sheet Pile | Area | Schedule | Control | Uncontrolled |  | Controlled |  |
|  | acres | days | percent | lbs/day | lbs/year | lbs/day | lbs/year |
| Bacon Island | 4.13 | 97 | 95% | 211 | 20,442 | 11 | 1,022 |
| Holland Tract | 4.13 | 97 | 95% | 211 | 20,442 | 11 | 1,022 |
| Connection Slough | 2.75 | 97 | 95% | 140 | 13,628 | 7 | 681 |
| Onsite Totals | 11.02 | 97 |  | 562 | 54,512 | 28 | 2,726 |
| Roberts Island | 4.13 | 20 | 95% | 211 | 4,215 | 11 | 211 |
| Offsite Totals | 4.13 | 20 |  | 211 | 4,215 | 11 | 211 |
| Sources: BAAQMD 1999, EPA 2006 |  |  |  |  |  |  |  |
| Notes: |  |  |  |  |  |  |  |
| Fugitive dust (as PM10) 51 lb/acre-day unmitigated, BAAQMD CEQA Guidelines, Section 3.3 |  |  |  |  |  |  |  |
| BAAQMD Ref: AP-42 Chapter 13.2.3 "Heavy Construction Operations" |  |  |  |  |  |  |  |
| Mitigation Ref: AP-42 Chapter 13.2.2 "Unpaved Roads", Figure 13.2.2-2 |  |  |  |  |  |  |  |
| Soil moisture ratio = 5 (for all feasible mitigation measures under project control) |  |  |  |  |  |  |  |
| Roberts Island soil moisture ratio = 5 (for permitted mitigation measures) |  |  |  |  |  |  |  |

Diesel Particulate Matter Emissions Control Measures

The following measures would be implemented to reduce particulate matter emissions from diesel exhaust:

- Grid power will be used instead of diesel generators where it is feasible to connect to grid power (generally contingent upon power line proximity, capacity, and accessibility).

- Specifications will include 13 CCR Sections 2480 and 2485, which limit the idling of all diesel-fueled commercial vehicles (weighing over 10,000 pounds, both California- or non California-based trucks) to 30 seconds at a school or five minutes at any location. In addition, the use of diesel auxiliary power systems and main engines will be limited to five minutes when within 100 feet of homes or schools while the driver is resting.

- Specifications will include 17 CCR Section 93115, Airborne Toxic Control Measure for Stationary Compression Ignition Engines, which specifies fuel and fuel additive requirements; emission standards for operation of any stationary, diesel-fueled, compression-ignition engines; and operation restrictions within 500 feet of school grounds when school is in session.

- A schedule of low-emissions tune-ups will be developed and such tune-ups will be performed on all equipment, particularly for haul and delivery trucks.

- Low-sulfur (maximum sulfur content of 15 ppm by weight) fuels will be used in all stationary and mobile equipment.

Construction emissions are transient and temporary, and BAAQMD and SJVAPCD control measures would be implemented as described previously. The Proposed Action would not expose sensitive receptors to substantial pollutant concentrations. The proposed sites are located in a sparsely populated rural (agricultural) area. The nearest house is approximately 600 feet (180 meters) south of the Old River site; however, it is unoccupied. The next nearest receptor is a marina with live-aboard boat owners approximately 0.8 mile south (1,300 meters) of the Old River site. It is not known whether the marina, which is outside the immediate vicinity (i.e., 1000 feet or 305 meters) of the Old River site, houses potentially sensitive persons.

Construction activities would cause short-term emissions of NOX, ROC, CO, SO2, PM10, and PM2.5 from diesel-powered equipment and earthmoving (ground disturbance). The results of the screening analysis contained in (b) above shows that no exceedence of ambient air quality standards in the vicinity of the Proposed Action would result solely from proposed activities. Notwithstanding impacts from the Proposed Action, maximum background levels of particulate matter (PM10, PM2.5) already exceed applicable state or federal standards.

Diesel particulate matter contain substances that are suspected carcinogens, along with pulmonary irritants and hazardous compounds that may affect sensitive receptors such as young children, senior citizens, or those susceptible to respiratory disease. Where construction activity occurs in proximity to long-term sensitive receptors, there could be a potential for unhealthful exposure of those receptors to diesel exhaust, including residential receptors. The results of the screening risk assessment contained in (b), analyses show that the probability of contracting cancer from diesel particulate matter, for the MEI is about 5 x 10-8, which is 200 times less than the 10 in one million (1 x 10-5) BAAQMD or SJVAPCD threshold.

California ultra-low sulfur diesel fuel with a maximum sulfur content of 15 ppm by weight would be used in all diesel-powered equipment which minimizes emissions of sulfurous gases (sulfur dioxide, hydrogen sulfide, carbon disulfide, and carbonyl sulfide). Moreover, the proposed sites are located in an unpopulated area, and the nearest area potentially containing sensitive receptors is approximately 0.8 mile from the Old River site. Therefore, no objectionable odors are anticipated from construction activities or normal operation of the Proposed Action.

“Controlled” means implementation of BAAQMD and/or SJVAPCD required emissions control measures. These measures are in Section 3.3.3.3.